Content

1.Overview	1

2.Operating instructions	2

2.1 Battery instructions	2

2.2 Switch on and off	2

2.3 Black and white calibration	3

2.4 Mobile APP installation and measurement	3

2.5 Quality management software installation	3

2.6 Measurement	3

2.6.1 APP measurement	4

2.6.2 Stand-alone standard sample measurement	5

2.6.3 Single machine sample measurement	5

3.Daily maintenance	6

4.Technical parameters	6

- 1.Overview

Mini ColorReader is a portable small color measuring instrument with small size, convenient use and accurate measurement. It is widely used in industries such as plastic electronics, paint and coating, textile and garment printing and dyeing.

The instrument can be connected to the mobile phone APP and SQCX PC quality management software to achieve more functional expansion. The mobile APP comes with multiple sets of electronic color cards, which can realize color card retrieval and matching. The mobile phone APP and SQCX PC software have a variety of light source, color space, color index, and reflectance functions, which can realize massive chromaticity index testing.

This instrument has the following advantages:

1) It can be measured after power-on, no need to perform white calibration every time, making color measurement easier;

2) Separate design of calibration whiteboard and measuring port to ensure long service life of whiteboard and accurate measurement;

3) Accurate, stable, small size, easy to carry, APP built-in mobile electronic color card, it is a good helper for mobile color quality control;

4) A variety of chromaticity parameters can be set on the instrument side, test records can be checked, and the stand-alone function is powerful;

5) Built-in rechargeable lithium battery, Type-C USB interface, easy to use;

6) Connect the mobile phone APP and PC SQCX software to achieve more functional expansion.

1. Button and interface description

Measuring Button

/Power Switch

Dispaly

Calibration Cover

n Cover

Type C

Measuring Aperture

Figure 1 Schematic

Note: You can also press the measurement button when the calibration box is placed above the measurement button

2.Operating instructions

2.1 Battery instructions

This instrument uses a built-in lithium battery, the specification of the lithium battery is Li-ion 3.7V, and the capacity is 800mAh. Use a USB data cable to connect to the USB port on the PC or connect to a 5V==1A power adapter to charge the lithium battery. The maximum charging current can reach 500MA. When charging, the battery level is greater than 80%, the LED indicator is green, the battery level is less than 20%, the LED indicator is red, and the battery level is between 20% and 80% is yellow. The display shows a charging prompt, and the charging is complete, indicating that the battery is full.

2.2 Switch on and off

Power on: In the power off state, press the power switch, the instrument is powered on, the LED indicator flashes, the display screen lights up and enters the measurement interface.

Shutdown: In the power-on state, long press the power switch to shut down the instrument. When the automatic sleep mode is turned on, the instrument will automatically shut down.

Note: (1) The instrument will automatically shut down when no one is using it for 10 minutes.

- When not using the instrument for a long time, please turn off the power.

2.3 Black and white calibration

Cover the calibration box of the instrument, and automatically calibrate after starting up.

2.4 Mobile APP installation and measurement

The mobile APP supports Android operating system version 5.0 and above. Use the mobile browser or WeChat to scan the QR code below, download and install as prompted, then turn on Bluetooth to search for connections. Please refer to the built-in manual of the APP for details on the operation steps and usage of the APP.

Figure 2 QR code for APP installation      Figure 3 App icon after installation

2.5 Quality management software installation

The instrument can establish a connection with the PC-side SQCX quality management software through a USB data cable or a Bluetooth module (only for products equipped with a Bluetooth module) to communicate and achieve more functional expansion.

2.6 Measurement

The standard sample measurement and sample measurement can be realized by the instrument, and the standard sample/sample switch can be switched by long pressing the measurement button and waiting for the buzzer to sound once.

And other chromaticity indexes can be realized by mobile phone APP and SQCX PC software.

2.6.1 APP measurement

After the installation is complete, open the mobile APP (see the mobile APP manual for details on the use of the mobile APP), click "Connect Bluetooth" or "Connect to the instrument", first check the instrument model and SN code in the "Device Information" of the instrument, and then in the mobile APP Select the instrument to be connected (instrument model: SN code) in the "Connected Instruments" list, and click to automatically connect. In the color measurement interface, click "standard sample measurement" or "take standard sample" to obtain the chromaticity data and simulated color of the standard sample. Click "Measure Color Difference" to obtain the chromaticity data, difference and artificial color of the sample, as shown in Figures 4 and 5.

Figure 4 Single machine measurement

Figure 5 Instrument measurement interface after connected APP

2.6.2 Stand-alone standard sample measurement

In the standard sample measurement interface, align the measurement aperture to the standard sample and press the "measurement button" shortly. The instrument obtains the chromaticity data of the standard sample, as shown in Figure 6. The green indicator LED flashes once when the measurement is completed. The status bar at the top of the instrument screen displays the current measurement conditions, the icon at the bottom displays the USB and Bluetooth connection status, and "01" in the lower right corner indicates the current standard number. The number of the stand-alone standard samples is from 01 to 10. When the standard sample data is full of 10, the system will prompt whether to clear the standard sample again. After clearing, the measurement can be continued. After the standard sample is cleared, the sample associated with the standard sample will also be cleared. Slide the operation button to the left or right to view the target data.

Figure 6 Instrument standard sample measurement

2.6.3 Single machine sample measurement

In the standard sample measurement interface, long press the "measurement button" to enter the sample measurement interface associated with the standard sample, as shown in Figure 7. Align the measuring aperture with the sample and press the "measurement button" shortly, and the instrument will obtain the chromaticity data of the sample. The green indicator LED flashes once when the measurement is completed. The “01” on the left in the lower right corner of the instrument screen represents the standard sample number associated with the sample, and the 01 on the right represents the current sample number, and the sample number ranges from 01-99. After 99 pieces of sample data are full, the system prompts whether to clear the sample again, and the measurement can be continued after the sample is cleared. Slide the operation button to the left or right to view the sample data.

The default DE tolerance of the instrument is 1.0. When the color difference is within the tolerance range, it will prompt "Qualified", otherwise, it will prompt "Bad". The instrument DE tolerance can be set via APP and SQCX.

Figure 7 Instrument sample measurement

- 3.Daily maintenance

1) This instrument is a precision optical instrument, please keep and use the instrument properly, and avoid using and storing the instrument in humid, strong electromagnetic interference, strong light, and dusty environments. It is recommended to use and store the instrument in a standard laboratory environment (temperature 20 degrees Celsius, 1 standard atmospheric pressure, humidity 50~70%RH).

2) The whiteboard is a precision optical component. It should be kept and used properly. Avoid hitting the working surface with sharp objects, avoid staining the working surface with dirt, and avoid exposing the whiteboard to strong light. Regularly clean the working surface of the whiteboard with a wipe cloth dipped in alcohol, and remove the dust on the working surface in time when correcting.

3) In order to ensure the validity of the measurement data, it is recommended that the complete instrument and the whiteboard should be inspected by the manufacturer or a qualified metrology institute one year from the date of purchase.

4) Please do not disassemble the instrument without permission. If you have any questions, please contact the relevant after-sales staff. Tearing the easy-to-tear label will affect the after-sales maintenance service of the instrument.

4.Technical parameters

| Optical Geometry | D/8 (diffuse illumination, 8° direction reception, including specular reflection light SCI)）, Conforms to standards CIE No.15，GB/T 3978. |
| --- | --- |
| Features | Used for color difference quality control in plastic electronics, paint and coating, textile and garment printing and dyeing, ceramics and other industries; Small size, easy to carry, search for the color cards and multifunction expanded through smartphone APP |
| Integrating Sphere Size | Φ20mm |
| Illuminant Device | Full spectrum LED light source |
| Spectro Method | Filter spectro（Some models do not have this function） |
| Sensor | CMOS sensor |
| Wavelength Range | 400～700nm |
| Measuring Aperture | Φ8mm |
| SCI/SCE | SCI |
| Color Space | CIE LAB, XYZ, Yxy, LCh, CIE LUV, s-RGB, HunterLab, βxy, DIN Lab99（Different models are different） |
| Color Difference Formula | ΔE*ab,ΔE*uv,ΔE*94,ΔE*cmc(2:1),ΔE*cmc(1:1),ΔE*00, DINΔE99,ΔE(Hunter) （Different models are different） |
| Other Color Index | WI(ASTM E313，CIE/ISO, AATCC, Hunter)， YI(ASTM D1925，ASTM 313), Metamerism Index Mt， Color fastness, color strength, Opacity, 555 color classification, Munsell(C/2) (implemented by mobile APP) （Some models do not have this function） |
| Display Accuracy | 0.01 |
| Observer Angle | 2/10°（Some models only support 10°） |
| Illuminants | D65, A, C, D50, D55, D75, F1, F2(CWF), F3, F4，F5，F6, F7(DLF), F8, F9，F10(TPL5), F11(TL84), F12(TL83/U30) （Different models are different） |
| Display | Sample chromaticity value, color difference value, pass/fail result, color simulation, color deviation, reflectance (partly through mobile APP) （Some models don’t support reflectance, Some models have no reflectivity） |
| Measuring Time | About 1.0s |
| Repeatability | Color：ΔE*ab within 0.05（After warm-up calibration, the average value of measuring the whiteboard 30 times at an interval of 5s） |
| Accuracy | Pass National Metrology(Some models are not guaranteed） |
| Measuring Mode | Single measurement，Average measurement（2~99times））（Implemented through smartphone App） |
| Locating Method | Spot positioning（Some models do not have this function) |
| Dimension | Ø30X100mm |
| Weight | 88g |
| Battery | Lithium battery, can do tests continuously for 12000 times in a single full charge |
| Light source lifespan | 5-years more than 3 million times measurements |
| Screen | IPS full color screen, 1.14inch |
| Port | Type-C USB；Bluetooth 5.0；Buttons |
| Data Storage | Mass storage can be expanded through mobile APP（Different models are different） |
| Language | Chinese，English |
| Working Temperature range | 0~40℃，0~85%RH（No condensation），Altitude：lower than 2000m |
| Storage Temperature Range | -20~50℃，0~85%RH（No condensation） |
| Standard Accessories | Data cable, manual, calibration box, PC Color quality management software, APP (download from official website) |
| Optional Accessories | USB mini printer, powder test box |
| Note | Technical parameters are for reference only, subject to actual instruments. |